Developing Real-Time Crash Prediction Model using Bayesian Network

Kh.M.Rifat Foysal

Department of Civil & Environmental Engineering

Islamic University of Technology

2015

Developing Real-Time Crash Prediction Model using Bayesian Network

Kh.M.Rifat Foysal

Student ID: 115424

A THESIS SUBMITTED FOR THE DEGREE OF SCIENCE IN CIVIL ENGINEERING

Department of Civil & Environmental Engineering

Islamic University of Technology

2015

APPROVAL

This is to certify that the thesis submitted by Kh.M.Rifat Foysal entitled as “Developing Real-Time Crash Prediction Model using Bayesian Network” has been approved fulfilling the requirements for the Bachelor of Science degree in Civil Engineering.

______________________________________

Moinul Hossain

Assistant Professor, 
Department of Civil and Environmental Engineering (CEE)

Islamic University of Technology (IUT)

Board Bazar, Gazipur, Bangladesh.

DECLARATION

I declare that the undergraduate research work reported in this thesis has been performed by me under the supervision of Assistant Professor Moinul Hossain. I have exercised reasonable care to ensure that the work is original and has not been taken from the work of others.

_______________________________

Kh.M.Rifat Foysal

ID: 115424

November, 2015

ACKNOWLEDGEMENTS

Writing a significant scientific thesis is hard work and it would not be possible without support. All praises to Allah (SWT) for giving me the opportunity to complete this report. I would especially like to thank my mother and father who have provided me with the strength and dedication needed to complete this thesis.

It is the proud privilege to acknowledge my sincere and deepest sense of gratitude to my respected Supervisor Moinul Hossain, Assistant Professor of Civil and Environmental Engineering for his untiring efforts, inspiration, invaluable guidance and constructive criticisms throughout the period.

I would like to express gratitude to all of the Departmental faculty members for their help and support.

I am also thankful to my friends and well-wishers for their valuable suggestion and cordial assistance.

ABSTRACT

Crashes are regular occurring phenomena and consequently seek attention as a lot of casualties, both lives and goods, take place as a result of these phenomena. Significant number of studies have been carried out to identify the proper measures to prevent crashes. Earlier studies in this field include determination of black spots and identifying crash patterns to build prediction models with historical data. Traffic flow variables such as ADT, hourly traffic volume or AADT, design speed limit etc. were used to develop models as a part of re-active traffic management. However, later studies have focused on instantaneous traffic flow to develop crash prediction models in real-time as a part of pro-active traffic management.

The concept for predicting crash events in real-time is still relatively new and researchers are paying substantial attention in this field of pro-active traffic management. The basic idea is to develop a real-time crash prediction model to identify crash-prone situation so that proper interventions can be provided in order to improve traffic safety on expressways. Many researchers have already put in considerable efforts to develop an effective crash prediction model with significant amount of accuracy and low false alarm. However, there are some major obstacles currently associated with these type of models that must be resolved in order to convert the theoretical arguments into probable actions in real life scenarios. For instance, the real-time crash prediction models use the different interactions among the variables to predict the existing traffic condition. So the models are built specific to a traffic condition within a specific segment of the expressway. This implies the necessity to have a model that can update itself in real-time with newly available traffic data in order to adapt to the traffic condition where it is being employed. Also given the rare nature of crash events, getting matching crash and traffic sensor data is difficult. Hence, an ideal model should be able to be built with relatively low sample size, must have the ability to update itself in real-time and must be capable to adapt to the traffic conditions. With a view to resolving these problems, this study proposes Bayesian Belief Net (BBN) as a platform to build a real-time crash prediction model. BBN as a probabilistic modelling method, allows to solve most of the major hindrances associated with real-time crash prediction model.

This study employs data from Shibuya 3 inbound route throughout the months of March, 2014 to August, 2014 provided by Tokyo Metropolitan Expressway Company Limited. The study route is 11.4 kilometer in length and has densely placed loop detectors that suites the requirements of the study to collect instantaneous traffic flow data. As the study is conducted on basic freeway segments only, so data are collected only from 42 specific loop detectors from the basic freeway segments. A total 5 minutes of data prior to crash events are utilized in this study. The traffic flow variables used are 1 minute aggregated avg. occupancy, 1 minute aggregated avg. speed, cumulative 1 minute vehicle count. Separate data bases for crash data and normal traffic data are created to develop 5 BBN models. Each of the BBN models is developed with variables from each of the minutes of total 5 minutes prior to the crash event. HUGIN Expert, a commercially developed software for BBN model building is utilized in this study. For minutes 1, 2 and 4 prior to the crash events, the generated BBN models identified specific influences of the variables on the crash events. The models are evaluated according to their performances. Evaluation criterion for the models are set to be accuracy, overall accuracy and false alarm rates. Model 1 and Model 2 has an overall accuracy of 70% and 58% with crash accuracy of 43% and 70% respectively. The non-crash accuracy of the Model 1 and Model 2 are found to be 83% and 53% with false alarm rates of 63% and 41% respectively. However, Model 4 is found to have fairly met all the criterion to be the best model with an overall accuracy of 60%, crash accuracy of 26%, false alarm rate of 40% and non-crash accuracy of 76%. The result of this study suggest that the newly developed model can successfully fulfill all the aforementioned requirements.

TABLE OF CONTENTS

| ACKNOWLEDGEMENTS |  | iv |
| --- | --- | --- |
| ABSTRACT |  | v |
| TABLE OF CONTENTS |  | vii |
| LIST OF TABLES |  | xi |
| LIST OF FIGURES |  | xii |
| CHAPTER 1 INTRODUCTION |  | 1-5 |
| 1.1 | Background | 1 |
| 1.2 | Problem Statement | 3 |
| 1.3 | Purpose and Objectives | 4 |
| 1.4 | Scope of the study | 4 |
| 1.5 | Thesis Outline | 4 |
| CHAPTER 2 LITERATURE REVIEW |  | 6-11 |
| 2.1 | Introduction | 6 |
| 2.2 | Use of Crash Prediction Models in Road Safety | 6 |
| 2.3 | Real-time Crash Prediction Models | 8 |
| 2.4 | Summary | 11 |
| CHAPTER 3 METHODOLOGY |  | 12-24 |
| 3.1 | Introduction | 12 |
| 3.2 | Study Area and Data Collection 3.2.1 Study Area 3.2.2 Experimental Setup 3.2.3 Data | 13 |
|  |  | 13 |
|  |  | 15 |
|  |  | 16 |
| 3.3 | Modelling method and Techniques 3.3.1 Bayesian Belief Nets (BBN) 3.3.2 Structural Learning: NPC algorithm 3.3.3 Batch Learning: EM Algorithm 3.3.4 Sequential Learning: Adaptation algorithm | 17 |
|  |  | 18 |
|  |  | 20 |
|  |  | 22 |
|  |  | 22 |
| 3.4 | Model Building 3.4.1 Variables 3.4.2 Models | 23 |
|  |  | 23 |
|  |  | 24 |
| CHAPTER 4 RESULTS AND ANALYSIS |  | 25-47 |
| 4.1 | Introduction | 25 |
| 4.2 | Preliminary Analysis of crash data | 25 |
| 4.3 | Data Preparation 4.3.1 Data Extraction 4.3.2 Aggregating the data 4.3.3 Congestion Index calculation 4.3.4 The Final Datasheet for HUGIN Expert | 31 |
|  |  | 31 |
|  |  | 33 |
|  |  | 33 |
|  |  | 33 |
| 4.4 | Model Development 4.4.1 Variable Selection 4.4.2 Model Construction with HUGIN Expert 4.4.3 BBN Models | 34 |
|  |  | 34 |
|  |  | 36 |
|  |  | 37 |
| 4.5 | Results of the Models | 42 |
| 4.6 | Evaluation | 43 |
| 4.7 | Classical problems with real-time crash prediction models and performance of Bayesian Belief Nets | 46 |
| 4.8 | Summary | 47 |

| CHAPTER 5 CONCLUSIONS |  | 48-50 |
| --- | --- | --- |
| 5.1 | Introduction | 48 |
| 5.2 | Objective Based Conclusion | 48 |
| 5.3 | Limitations and Future Scope | 50 |
| REFERENCES |  | 50-57 |

LIST OF TABLES

| Table |  | Page |
| --- | --- | --- |
| 1 | Data collection summarization | 17 |
| 2 | Variables and their description | 23 |
| 3 | Crash Data Obtained for the study | 26 |
| 4 | Summarization of Spatial Distribution of Crashes | 30 |
| 5 | Identification of detectors in respect to Crash IDs | 31 |
| 6 | A sample of the preliminary data sheet | 32 |
| 7 | A part of the final data sheet | 34 |
| 8 | Summarization of selected variables | 35 |
| 9 | Summary of the results | 42 |
| 10 | Results from Case 2 | 43 |
| 11 | Evaluation of MODEL 1 | 44 |
| 12 | Evaluation of MODEL 2 | 44 |
| 13 | Evaluation of MODEL 4 | 45 |
| 14 | Summary of Evaluation | 45 |

LIST OF FIGURES

| Figure |  | Page |
| --- | --- | --- |
| 1 | Flow chart for the development procedure of the model | 12 |
| 2 | Study Area | 13 |
| 3 | Detailed map of the study area | 14 |
| 4 | Experimental Setup | 15 |
| 5 | An example of Bayesian Belief Net | 19 |
| 6 | Distribution of crash types | 27 |
| 7 | Monthly Distribution of Crashes | 28 |
| 8 | Weekly Distribution of Crashes | 28 |
| 9 | Hourly Distribution of Crashes | 29 |
| 10 | Spatial Variation of crashes | 30 |
| 11 | BBN MODEL 1 | 37 |
| 12 | BBN MODEL 2 | 38 |
| 13 | BBN MODEL 3 | 39 |
| 14 | BBN MODEL 4 | 40 |
| 15 | BBN MODEL 5 | 41 |

CHAPTER 1

INTRODUCTION

  - Background:

For improving the safety of the expressways by preventing traffic crashes, real-time crash prediction models are attracting substantial recognition from the researchers all around the world since the past two decades. Most of the crashes on expressways take place as a result of complex interaction of human, vehicle, and environmental factors with the geometric configuration of the roads as per the findings of Treat et al. (1977). But in later studies conducted by Oh et al. (2001),      it was hypothesized that although the human interaction, vehicle factors, environmental factors and road geometry remain suitable to the drivers, crashes can still occur if the existing traffic condition suddenly changes. For their reasoning, they proposed a new variable contributing to crash incidents namely the traffic dynamics. The sudden change in traffic formation is nothing but the transition of normal traffic condition into a hazardous traffic condition which is prone to crashes – as proposed by them.

The basic idea of real-time crash prediction is to collect traffic flow data in real-time and categorize them as either hazardous or safe based on a model built with previous knowledge gained from archived traffic flow and corresponding crash data. For developing such real-time crash prediction models, almost invariably high resolution sensor data are being used. Researchers have put significant effort on improving the prediction rate with low false alarm ( Hossain and Muromachi, 2012, Yu and Abdel-Aty, 2013, Xu et al., 2013) and understanding crash phenomena using the model outputs (Hossain and Muromachi, 2013, 2011 and Xu et al., 2012 ). Some researchers also studied possibilities of interventions to bring a hazardous traffic condition back to normal for preventing crash. Their choice of interventions were variable message signs (Lee, C., and M.Abdel-Aty, 2008), variable speed limits (Hellinga , B., M. Mandelzys, 2011,M. Abdel Aty et al., 2008), ramp metering (  Abdel-Aty, and Gayah,2010) etc.

In the past 30 years, researchers spent considerable amount of time into identifying factors influencing crash (Fridstrom et al., 1995; Miaou and Song, 2005) and developed crash prediction models to calculate the frequency and the severity of the crashes (Khan et al., 1999; Caliendo et al., 2007). For the calculation and analysis of crash frequency in the road segments in the expressway, aggregated crash data of the specific roadway segment used to be employed. Donnell and Mason (2006) divided the roadway into ½-mile segments for their analysis of crash frequency. Oh et al. (2001), one of the pioneers of the concept of real-time crash prediction, hypothesized that a 5 min standard speed deviation value can be taken as a pointer as to when a hazardous traffic flow condition is gradually leading to a potential crash occurrence. This marks the dawn of a new paradigm in crash prediction which extends the conventional idea of associating crash frequency with a specific road segment for a large period of time in the future to calculating probability of discrete crash events for small road segments for a very short span of time in future, in most cases, less than 10-15 minutes. The idea was later supported by several researchers as (Lee et al. 2002, 2003; Pande & Abdel Aty 2005) they themselves proved the hypothesis that by using the instantaneous traffic flow data, crash on a specific segment of highway can be predicted.

Abdel Aty et al. (2004) considered the spatial variation and developed crash prediction models using case control logistic regression. With the contribution of several researchers (Lee et al., 2002, 2003; Abdel-Aty and Pande, 2004; Abdel-Aty and Abdalla, 2004; Oh et al., 2005; Abdel-Aty et al., 2006; Dias et al., 2009; Hossain and Muromachi, 2010), the idea of developing crash models has taken another step forward from the theoretical stage towards the real-life scenario where such models can be used for proactive road safety management. In recent times, the studies on real-time crash prediction are expanding from expressways or freeways as Jang et al. (2012) have taken the opportunity to introduce a real-time crash warning system for intersections where conditions related to vulnerable line of sight and/or traffic violation can be evaluated. The latest contribution to proactive road safety management system also considered visibility related crashes (Hasan, Hany M., M. Abdel-Aty, 2013), inclement weather effect (Sun et al., 2011), roles of exposure and speed (Pei et al., 2012), crash prediction models for road tunnels (Ciro Caliendo et al., 2013) etc. In a later study Rongjie Yu and Abdel-Aty (2013) worked with a view to differentiating the traffic flow conditions that affect the crash pattern on weekday and on weekends and developed a real-time crash prediction model to confirm it.

1.2 Problem Statement:

There are some inherent hindrances associated with existing real-time crash prediction model development which needs to be overcome for an applicable solution that can be implemented in a real-life scenario. As such a model is highly dependent on sufficient number of crash data having matching sensor data. As crash is a rare event most often it does not become possible to build such a model with sufficiently large sample size.

Different types of sensors collect different types of data, e.g., all kinds of sensors cannot capture speed data or occupancy data accurately enough. Moreover, all sensors do not have the capability to capture data of all three basic variables—speed, flow and occupancy. A reliable modeling method for real-time crash prediction is needed that should be able to handle missing data. The model also should have the ability to incorporate new variables without having to build it from the scratch.

It is expected have the capability to make prediction when information on all the variables is not available. Finally the existing models are build based on the available detector spacing and traffic condition specific to the concerning expressway. This emphasizes the importance of having a model that have the ability to update itself in real-time and customize itself so that it matches with traffic condition of the expressway where it will be employed, i.e., they must be transferrable. Considering these requirements, this study proposes Bayesian Belief Net (BBN) (Jensen et al., 2007 and Kjaerulff et al., 2008) as a platform to build real-time crash prediction models.

BBN is a probabilistic graphical modeling method that describes complex joint distribution of a system through a graph and local distributions, i.e., conditional probabilities. It has an integrated system to handle with the missing data while developing the model. It can also incorporate new variables and update itself accordingly even after it is fully built. Moreover, it has the inherent capability to calibrate itself with partially available data. It supports sequential learning, which allows it to update itself in real-time whenever new data becomes available.

  - Purpose and Objectives:

The purpose of this study is to present BBN as a platform for real-time crash prediction model building. The specific objectives are:

- Develop real time crash prediction models with BBN by applying Necessary Path Condition (NPC) algorithm for structural learning and Expectation-Maximization (EM) algorithm for parameter estimations of conditional probabilities;

- To evaluate the performance and accuracy of the developed real-time crash prediction models

  - Scope of the Study:

The study area is required to be pre-equipped with sensors yielding data for short time intervals that characterize traffic status such as speed, flow or occupancy. Moreover, it is expected that the study area have closely spaced sensors for accurate data. Additionally, the study requires sufficient number of corresponding crash samples.

  - Thesis Outline:

The thesis is composed of 5 chapters.

Chapter 1 includes the background of the study, problem statement, purpose of objective and scope of the study.

Chapter 2 is the brief overview of the related studies, their advantages and short comings for a better understanding of the significance of the thesis.

Chapter 3 provides with an in depth discussion of the methodology that has been employed to conduct the study. It deals with the understanding of the fundamental ideas and concepts such as BBN, several algorithms put to different uses. Also this chapter contains the work flow as well as a detailed description of study area, data collection and model building.

Chapter 4 is a comprehensive description about the analytical part of the study. It demonstrates the various steps in the analysis part such as data preparation as well as the model developing sequences. Also the outcomes of the study is also explained in details in this chapter.

Chapter 5 is an overall discussion of the study in light of the findings from the previous chapters. It also encompasses the effectiveness of the thesis and comments on the results.

CHAPTER 2

LITERATURE REVIEW

2.1 Introduction:

Over the last couple of decades, the necessity for a proactive road safety management system has increased a lot due to the heavy number of crash events. As a new approach to contribute to traffic safety, real-time crash prediction models are given substantial priority among the transportation researchers. Therefore, a great number of researches have already taken place with the primary aim to develop a real-time crash prediction model. However, still real-time crash prediction models are considered relatively new in the field of proactive road safety management system. Although some advancements have been achieved in building models that can predict crashes in real time, the challenge remains to develop a model with low false alarm rates and with the capability of being implemented in real life. As the primary aim of this thesis is to develop real-time crash prediction models, this chapter discusses the existing studies related to this field of road safety. The chapter also summarizes the use of crash prediction models in road safety. A thorough literature review related to the thesis topic is conducted and the major findings and short comings from the existing works have been documented in this chapter of the thesis.

2.2 Use of Crash Prediction Models in Road Safety:

The use of crash prediction models in road safety goes back a long time when Chapman (1973) used them to estimate the probability of crashes in a given time in a given area. He defined the predetermined area and time as exposure and proposed a ration of the number of crashes to the exposure. Later Caliendo et al., 2007 contributed to these models.

For the crash prediction models, the primary priority was given to identify hazardous or high risk locations on the roadways (Abbess et al., 1981, Heydecker et al., 2001). There are some studies that also used crash prediction models to determine the influences of the variables or indicators of crash events (Golob et al., 2004; Miaou et al., 2005).

A policy related study was done (Miaou et al., 2005) where crash prediction model was used to adopt a suitable budget. The idea was to predict crash events and carry out an assessment of the damages that may occur. Later on the basis of a cost benefit analysis, the required resource allocation was determined. Also Federal Highway Administration in the USA uses crash prediction models in a similar manner to test their future roadways in respect of geometric design (Ogden, 2007).

Brodsky (1988); Keay and Sommonds, (2005, 2006); Bossche et al., (2006), Schalkwyk et al., (2006) used calendar data to develop crash prediction models. Their researches were conducted to identify the effects of seasonal variation of crashes. Comparing to previous models it was found that these models could provide similar outputs but the results showed varying crash rates for different seasons. So this opened the path to further researches emphasizing on the effects of rainfall, high temperature, snowfall etc. on crash events. Later Bossche et al., (2006) found that these models can help when AADT data is not available because traffic volume is supposed to have an internal relationship with traffic flow.

Gwynn (1967) was the first to propose a hypothesis that hourly volumes would be directly related to crash rates. Ceder, (1980), Ceder and Livneh, (1982) conducted an in-depth study to understand the correlation of crash events and traffic flows with hourly flow data. They used a very large data base over a period of eight years.

Lord (2000) proposed a paradigm shifting concept in the field of crash prediction models. He used micro simulation models to generate future traffic flow in order to find out the safety of the roadways. From his study it was observed that the success of the model to predict the safest paths depended on the accuracy of the predicted future flow. It was then well established that the accuracy of the crash prediction models are deeply interconnected with the accuracy of predicted future flow as well as the historical traffic flow data. It is considered that the idea of real-time crash prediction models generated from this concept.

Crash prediction models developed in the conventional ways used variables like AADT, road geometry, functional road classification and traffic control systems. Several methods were also employed to develop the crash prediction models. Hadi et al., (1995), Lord et al., (2005); Lord et al., (2006), in their studies, used the generalized linear models. Bayesian approach was adopted to develop crash prediction models by Hauer, (2001); Miaou et al. (2005); Persaud et al. (1999); Qin et al. (2005). Among these models Poisson and negative binomial distribution based models were found to be more acceptable considering their ability to control rare events as crash is considered to be rare event. Miaou (1994) conducted a study where the comparison between these two approaches were vastly discussed. Later, Abdel-Aty and Pande (2007) considered these as “collective approach” to estimate the performance of a road section.

2.3 Real-time Crash Prediction Models:

The concept of real-time crash prediction model began with the work of Dingus, et al. (1999) just at the beginning of 21st century. They developed two techniques in identifying the safety impacts in Intelligent Transportation System. In their work, first they employed a technique to identify the chance of isolating near miss crashes and driving errors in real time. And followed this by the second technique to use these data as inputs for assessing safety impacts. But the first research specifically mentioning the concept for real-time crash prediction model was conducted by Oh et al.,(2000,2001). Oh et. Al (2001) took a Bayesian approach to estimate the crash likelihood in real time. Their idea was to incorporate advanced traffic management and information systems in identifying crash prone situations. Their study area was I-880 freeway. The objective of the study was to develop a real time crash prediction model. They collected real time traffic data from inductive loop detectors. Their concept was developed on the basis that prior to crash, an unstable traffic conditions occurs which is also disruptive. They wanted to present such conditions with spatial and temporal variations in significant parameters. The variable they used was the standard deviation of speed. They used t-statistics to identify the key variable. They concluded that reduction in speed variation may decrease the crash probability. Some key assumptions were made due to lack of data. That also limited the scope of the study. Later, two more studies were conducted by them (Oh et al., 2005a, 2005b) using the same dataset but changing the methodology to improve and compare results.

Golob, T. and Recker, W. (2001) took an approach to measure the relationships between the crash types on freeways and the pattern of traffic flow. In addition to this, they also incorporated the effect of weather and lighting conditions in their study. But one thing is that instead of trying to predict crashes they used real-time traffic flow data to identify the crash patterns. Their study area was I-5, State Route 22, 55, 57, 91 and 405, CA. They used loop detectors to collect data. They used a number of variables: flow, occupancy, their lateral difference and also took into account the effect of weather and lighting. Their modelling method was non-linear canonical correlation analysis. In spite of their using real-time traffic flow data, the main purpose of the study was to identify crash patterns.

Lee et.al (2002) proposed a study where they called for proactive measures rather than reactive measures and their study reflected this view. They used the term “crash precursors” for the characteristics that are observed just prior to the crash. They tried to find the significant variables that may lead to potential hazardous traffic condition that can ultimately result in a crash occurrence. To understand the relationships between traffic conditions and the crash occurrences, they picked their study area as Gradinar expressway, Toronto. Their effort was to relate coeff. of variation of speed at the upstream, avg. density at the upstream, avg. speed difference between the upstream and downstream, freq. of crash, exposure in veh.-km with road geometry to understand the relationship of this variables with crash phenomenon and predict crash in real time. They used aggregated log-linear model for their research. In their research, they found that speed variations and occupancy was significant variables. Their model output was the probability of real-time crash events.

After that, several studies were conducted on the development of real-time crash prediction models. A number of researchers (Abdel-Aty and Pande, 2004, 2005; Abdel-Aty and Abdalla, 2004; Abdel-Aty et al., 2004, 2006, 2007; Pande and Abdel-Aty, 2005, 2007; Abdel-Aty and Pemmanaboina, 2006) have spent a considerable amount of time in developing real time crash prediction models with different experimental settings, variables and methods. For example, Abdel-Aty, M. and Pande, A. (2004) conducted a study to identify the patterns in the traffic flow data collected from loop detectors to predict crashes. Their concern was to improve the performance of Advanced Traffic Management Centers. They used the data from Interstate-4 from Orlando. They used Probabilistic Neural Network which is an implementation of multivariate Bayesian classifier along with Parzen estimator. The variable used by them was coefficient of variation in speed at the downstream and average occupancy at the station upstream. They implemented match case-control logistic regression. In essence they categorized the crash prediction results as traffic conditions leading to or not leading to crash.

But a problem remained in case of transferability of the models. Abdel-Aty, M. and Pande, A. (2006) tried to address this problem where they proposed to develop a real-time crash prediction model with emphasizing on the implementation. They used the same dataset from Interstate-4 from Orlando corridor for the period of 1999-2002. A simple model with binary outcome was first developed involving only one co variate. They incorporated speed variation, average occupancy and standard deviation of volume as significant variables in a multivariate logistic regression model. They projected different scenarios to help understand the crash prone traffic patterns so that impacts of crash can be minimized by notifying the crash mitigation squad. They also provided a two-stage real-time application strategy for helping the implementation of crash prediction model in mitigation of crash occurrences. Following that, Pande A., M. Abdel-Aty (2009) took an approach to assess the crash patterns on multi lane arterial segments. They categorized the crashes into four groups:  severe rear-end, lane-change related, pedestrian, and single-vehicle/off-road crashes. Their study area was multilane arterials in the state of Florida and they collected a random sample of time, day of week, and location (i.e., milepost) combinations. They used case control sampling for the modelling of the data. Here emphasize was given on roadway geometry rather than traditional crash precursors. They also identified severe and non-severe crashes on the selected road way segments to predict crash patterns.

The researchers also studied the different types of collisions such as lane-change related collusions (Pande and Abdel-Aty, 2006a), side-wipe crashes (Lee et al., 2006) or rear-end crashes (Pande and Abdel-Aty, 2006b). Also comparing the approach of real-time and non-real-time models (Abdel-Aty and Pande, 2007) and transferability of the models (Abdel-Aty et al., 2008) were conducted.

In more recent times, some notable studies were conducted by Hossain, M. and Muromachi (2011) where they employed a Bayesian approach on basic freeway segments to develop a real-time crash prediction model. They used random multinomial logit models to find the important variables and the significant detectors for data extraction. Later Hossain, M. and Muromachi (2012,2013) they followed up their work with two more researches where they employed the same Bayesian approach in developing crash prediction models and used high resolution traffic data. In one study, they identified ramp vicinities as potential crash spots in the expressways taking the driving errors that may occur while passing over this spots into account and developed a real-time crash prediction model. In the latter study, developed a real-time crash prediction model to identify traffic patterns that may develop before a crash event and tried to understand the reasons behind this mechanism. Xu et.al (2013) proposed a study to enhance the quality of the real-time crash risk prediction models in case of spatial and temporal transferability. They adopted the Bayesian method to update the models in real time. They concluded that the Bayesian updating approach can actually enhance the performance of the new updated models. Both temporal and spatial transferability were improved with very small data requirement.

Ahmed, M.M. et al (2014) implemented Bayesian logistic regression model to determine the crash patterns in foggy weather. They used Bayesian logistic regression to determine the effects of reduced visibility on crash events. They conducted their research with the use of airport weather information, roadway characteristics and aggregated traffic parameters.

Another advancement in the field of real-time crash prediction is made by Sun, Jie and Sun, Jian (2015). In their research, they applied Dynamic Bayesian Network to identify crash events using dynamic speed data. They also categorized the traffic conditions near the crash sites in several state conditions. And with the DBN as modelling tool, they achieved significance accuracy in their research.

Roshandel, S. et al (2015) conducted a study where they systematically reviewed the existing real-time crash prediction models and identified various techniques that have been employed by the researches in this field of science. They also presented a qualitative summary of the models and ran a meta-analysis of those studies.

2.4 Summary:

A careful study of this chapter provides insights on the history of crash prediction models. A brief understanding of the crash prediction models and how they have evolved over the years are elaborated with the reference of various researches in this field. The introduction of real-time crash prediction models are carefully illustrated and the shortcomings and the findings of existing researches have been outlined. The more advanced studies in the field have been described briefly in this part of the thesis. As the aim of this research is to present Bayesian Belief Net as a platform for real-time crash prediction model, the various researches that have already been conducted using BBNs are also mentioned in the literature review.

Chapter 3

Methodology

3.1 Introduction:

The purpose of this thesis is to present Bayesian Belief Net (BBN) as a platform for developing a real-time crash prediction model. It involves a sophisticated methodology specifically targeted to identify hazardous traffic condition prior to the crash events. To develop such a model, first a comprehensive understanding of BBN is required. Also there are several modelling techniques and algorithms that have been implemented in this study to achieve the specific objectives. In this chapter, the data collection methods as well as the experimental setup that has been adopted will be demonstrated. After that, there will be an in-depth discussion of BBN. Then algorithms used for structural learning will be discussed along with the batch learning algorithms. And then the variables used for the thesis and the model building methods will be described before proceeding to the analysis. Figure 3.1 represents a step-by-step development procedure of the proposed model:

Figure 3.1: Flow chart for the development procedure of the model

For keeping with the flow chart, this chapter will proceed to the selection of variables and setting terms for drawing the network. We will elaborate on how to work with BBNs as well as the collection of data. The other steps of the flow diagram will be covered in the latter chapters.

3.2 Study Area and Data Collection:

3.2.1 Study area:

To achieve an acceptable outcome from this study, it is necessary to find a suitable study area where large numbers of crashes occur in a regular basis. The study area needs to be well equipped with a large number of detectors as well. Moreover, it is required that the spacing between the detectors is fairly uniform. Shibuya 3, a route of Tokyo Metropolitan Expressway, Tokyo, Japan has been selected for conducting our research. This is an expressway that most suites the study requisites to have a substantial amount of densely spaced detectors. So it has an excellent built-in capability to collect high resolution traffic flow data as well as crash data. Figure 3..2 represents the study area:

Figure 3.2: Study Area

As it is one of the busiest urban expressway of Tokyo, a lot of crashes occur in here throughout the year. It has two lanes in each direction. The length of this expressway is 11.9 kilometers connecting areas namely Minato, Shibuya, Meguro and Setagaya. There are 99 detectors present in the Shibuya expressway with an approximate spacing of 250 meters. But 42 detectors are used to collect data for this study. Figure 3.3 is presented here containing a detailed map of our study area with the positions of detectors:

Figure 3.3: Detailed map of the study area.

Source: provided by Tokyo Metropolitan Expressway Company Limited

3.2.2 Experimental Setup:

For this study, it is required to extract data from loop detectors placed throughout the study area. But first we must select the detectors from which we will collect data. For this study, only the Basic Freeway Segments are selected. Hossain, M., and Y. Muromachi (2012) have used random multinomial logit models to establish that the detectors 250 meters apart from the crash location both in upstream and downstream to be the most important sensors for the Basic Freeway Segments. In this study, their methodology is followed. 42 specific detectors from the basic freeway segments were used to collect the data. Both in upstream and downstream, the detectors have been chosen in approximately 250 meters apart from the crash site.

The experimental setup can be illustrated in the following figure 3.4:

Figure 3.4: Experimental Setup

From the illustration of our experimental setup, the chosen detectors can be identified. They are:  one detector in each lane in just upstream of crash location and one detector in each lane just downstream from crash location.

3.2.3 Data:

Considering the main purpose of the study, for understanding crash phenomena using real-time crash prediction model, crash data base as well as traffic flow data base are needed to be prepared for the study area.

The traffic flow database is supplied by Tokyo Metropolitan Expressway Company Limited. The detectors placed in the study area collect and store the speed, occupancy, vehicle count and number of heavy vehicles for each 8 milliseconds throughout the year (24 hours a day, 365 days a year).

Every lane has a detector, so every detector position has two detectors, one in each lane. The collected and stored data gets aggregated for one minute for each of the detector points. Therefore the data base provided to us contain cumulative vehicle counts, cumulative heavy vehicle counts, average speed and average occupancy for each of the detector points.

A total of 230 crash data were collected for crashes occurring in Shibuya 3 inbound route throughout the months of March, 2014 to August, 2014. These crash data provide information on date, weekdays and weekends, time (minutes), location of crash in respect to kilo posts in nearest 10 meters (direction and position), vehicles involved, crash type, crash lane, etc. This data base is called the crash data base for this study.

A total 5 minutes of data, of which 4 minutes are prior to the crash, are collected from the traffic flow database corresponding with the crash locations and time gathered from the crash database for the 230 crashes that occurred in the six months. Aggregated data of 1 minute of cumulative vehicle counts, average speed and average occupancy data are extracted. These data are called crash data. Here, data are collected for 5 minutes in total but data for the individual minute of those 5 minutes are aggregated.

In addition to that, 5 minutes of data are also collected in the same manner from randomly chosen 2 days of the same month (in which days no crashes occurred).  These data are defined as normal traffic data thus distinguishing it from the earlier crash data. For the 230 crash cases, 460 normal traffic data have been collected.

Table 3.1 presenting the summary of various data collection is provided here:

Table 3.1: Data collection summarization

| Data Type | Parameters |
| --- | --- |
| Detector Data | For 1 minute: Speed, Flow, Occupancy and Number of heavy vehicles – summed up for all lanes. |
| Crash Data | Date, time, location, vehicles involved, location of crash in respect to kilo posts in nearest 10 meters (direction and position), types, lane |
| Normal Traffic Data | Randomly selected traffic data from 2 days in which no crashes have occurred in the same month as the crash occurrences |
| Map | Location of ramps, detector positions, section length. |

3.3 Modelling method and Techniques:

In this part, the various modelling methods and techniques adopted to develop the models are elaborated for better understanding:

3.3.1 Bayesian Belief Nets (BBN):

Bayesian Network is also known as Bayesian Belief Nets. It provides a graphical structure with probabilistic distribution in an uncertain domain. It defines a system with many variables and possible outcomes that can be presented as a graph where the conditional independences are asserted with compacted joint probabilistic distribution.

The BBN consists of both qualitative and quantitative parts.

In the qualitative parts, the followings can be noted as syntax:

- Directed Acyclic Graphs (DAG): As the name points out, there should be no cycling back in the directed acyclic graphs. It means that if we have a directed graph, the path in the graph cannot come back to the starting node or vertex.

- Nodes: Nodes in the BBN represent the variables present in the system. When one node generates a direct influence on another, the node generating influence is called the parent node, and consequently the node being influenced is called the child node.

- Edges: Edges are the arcs that connects the nodes based on the influence of the one variable on others. It should be noted that the influence may or may not represent causality.

The quantitative part includes:

- Conditional Independences:

This part describes how the chain rule representation and the joint probability distribution can be incorporated in producing BBNs by applying Bayes’ Rule.

- Probabilistic Inference:

Inference provides ways to minimize the computational and space complexity.

For further explanation, Figure 3.5(a) can be taken into consideration where a BBN involving five variables are presented:

Figure 3.5: An example of Bayesian Belief Net

Here A is parent to D, D is parent to E, B is parent to A and E, C is parent to E. Also nodes C and E are not parents to any nodes. For general cases, a set of variables M= (m1, m2, m3………… mn-1) can be considered. Now from chain rule of probability using the joint probabilistic distribution, it can be written as:

PM= i=1nPmim1…mn-1 (3.1)
  
                                                                                                                                               
From Bayes’ Rule, this joint probabilistic distribution can be compacted and the equation (3.1) can be rewritten in this form:

PM= i=1nP(mi|parents(mi)) (3.2)
PU= i=1nP(Ai|Pa(Ai))

Here a DAG consistent with ordering is picked and represented in accordance with Bayes’ Rule. And it should be noted that it can reduce the number of conditional probability tables in a substantial manner.

Now some inference can be run with the help of variable elimination which will be done by marginalization. Now, if there is evidence e1… ea. on a number of variables out of n (n>a), the equation 3.2 can be written as:

PM,e= i=1nP(mi|parents(mi))j=1aej                                                                                   (3.3)

After reaching to this point in the argument, it can be stated that the probability of any variable m from the variable set M= (m1, m2, m3………… mn-1), can be calculated by marginalizing PM,e as follows:

Pme=M\mP(M,e)P(e)= M\mP(M,e)mP(M,e)                                                                                           (3.4)

Hence, the BBN from Figure 3.5(a) can be written as:

P(A,B,C,D,E) = P(A|B)P(B)P(C)P(D|A)P(E|B,C,D)	                                                                (3.5)

One of the desirable quality of the BBN is that here new variables can be added without needing to build the model up from scratch. And if a new variable is added to the system presented in Figure 3.5(a), it will generate the Figure 3.5(b). Now the newly developed BBN can be expressed mathematically as:

P(A,B,C,D,E,F) = P(A|B) P(B|F)P(C|F)P(D|A)P(E|B,C,D)P(F)                                               (3.6)

An observation can be made that the addition of new variable F can successfully update the model, all the while keeping the original model almost unchanged.

    - Structural Learning: NPC algorithm:

Structure learning basically means identifying and establishing relationships among the different variables present in a certain system when the interaction among the variables is unknown. In other words, structural learning enables us to determine the direction of edges from one variable to another one or more variables of the system. There are several structural learning algorithms which can be employed to draw up the BBN such as search-and-score algorithm and constraint-based algorithms. For our research, we chose a constraint-based algorithm namely the NPC (Necessary Path Condition) algorithm which originates from PC algorithm.

The main working principle of both algorithms is to run numerous statistical tests in order to determine the dependency relationships among the variables.

First, an elaborate discussion about the PC algorithms is required to get a better grip on understanding the NPC algorithms. PC algorithms perform the followings:

- They run statistical tests for conditional dependences on all the available variables in the system.

- After that a skeleton of the structure is produced where some of the variables can be directed and others remain undirected as conditional dependence cannot be determined.

- To ensure the acyclic nature of the directed graph, Colliders (when two variables meet in a common variable resulting in a cyclic order) are identified and resolved.

- The existing Colliders and positively identified conditional dependences are then put to use to establish directional edges.

- Lastly, still remaining undirected links are given direction randomly all the while making sure that the BBN remains a DAG.

PC-algorithm performs well when there is no limitation in sample size. In low sample size, problem can arise. Specifically, some inconsistencies can be observed in the conditional dependence statements. To resolve this issue NPC can be introduced:

“the necessary path condition for the absence of an edge says that in order for two variables X and Y to be independent (in a DAG faithful data set) conditional on a minimal set SXY, there must exist a path between X and every Z ∈ SXY (not crossing Y) and between Y and every Z ∈ SXY (not crossing X)”

NPC provides a way to interact with the ambiguous regions where no decisive directions can be established and researchers can identify the directions through their expert opinion to resolve the problem.

3.3.3 Batch Learning: EM Algorithm:

Parameter estimation in case of missing values in BBNs can be performed by employing EM algorithm (Expectation-Maximization algorithm). It involves the estimation of maximized likelihood of the incomplete or missing data.

The algorithm has two steps –

- Calculating the expected sufficient statistics,

- Then maximizing its likelihood.

To elaborate more, if no probability is assigned to a variable for which we are estimating the parameters, a uniform distribution is assumed. Then, with presence of a batch of data, the new parameter is estimated in such way that first, the expected sufficient statistics under that parameter is calculated and then the log-likelihood of that parameter under the expected sufficient statistics is maximized.

This is an iterative process and it stops when one of these two criteria are satisfied –

- The maximum number of iteration specified by the user has exceeded, or,

- The relative log-likelihood between two successive iterations is smaller than the preset minimum difference value

Here, it is important to mention that the EM-algorithm does not need data on each of the variables to update the model.

    - Sequential Learning: Adaptation algorithm:

Sequential learning is very important in the production of a model when parameters need to change over time to adapt with the new conditional probability distribution. It works almost in the same process as the EM algorithm. The difference being, here the evidence from each record is propagated throughout the network and the parameters for each of the variables are updated accordingly.

3.4 Model Building:

In this section, the selected variables for the model building will be discussed. Also the model characteristic will be defined.

3.4.1 Variables:

Traffic flow variables have been selected for the construction of real-time crash prediction model in this study. So the variables are mainly the speed, flow and occupancy.

In addition to that, a new variable is introduced namely “Congestion Index”.

Congestion Index (CI) = (Free Flow Speed - Speed)/Free Flow Speed;

When CI>0 = 0; when CI<=0.

The free flow speed is calculated from speed-flow and speed occupancy diagram of the study area and CI is estimated.

A detailed list of the variables used with their description is summarized below in table 3.2:

Table 3.2: Variables and their description

| Variables Used | Description |
| --- | --- |
| U_S | 1 minute aggregated avg. speed in upstream detector |
| U_O | 1 minute aggregated avg. occupancy in upstream detector |
| U_F | Cumulative 1 minute vehicle count in upstream detector |
| D_S | 1 minute aggregated avg. speed in downstream detector |
| D_O | 1 minute aggregated avg. occupancy in downstream detector |
| D_F | Cumulative 1 minute vehicle count in downstream detector |
| CI_U | 1 minute aggregated Congestion Index in upstream detector |
| CI_D | 1 minute aggregated Congestion Index in downstream detector |

3.4.2 Models:

A total of five models will be developed to understand the crash phenomena and to evaluate the performance of the real-time crash prediction models.

Five BBN models for real-time crash prediction will be developed, namely, Model 1, Model 2, Model 3, Model 4 and Model 5.

For developing these models, following considerations will be done to evaluate the performances:

The following situation will be considered where P (crash| D_O, D_S, D_F, U_F, U_S, U_O, CI_U, CI_D) will be calculated for minute 1, minute 2, minute 3, minute 4 and minute 5 prior to the crash events.

Next, Models will be developed by applying EM-algorithm to update themselves with the evaluation data.

The models will be evaluated with these two cases:

- Case 1: Here, the models will evaluate each of the remaining crash data and randomly selected normal traffic condition representing data from the remaining dataset and measure the prediction capability of the models by calculating P( crash | D_O, D_S, D_F, U_F, U_S, U_O , CI_U,CI_D)

- Case 2: Alternately each of the remaining crash data and normal traffic condition data will be entered into the models to predict P ( crash| D_O, D_S, D_F, U_F, U_S, U_O , CI_U,CI_D)  Hence, for Case 2, a situation will be simulated where the models are being updated in real time.

Case 1 will generate the prediction capacity of the models and the performance can be evaluated by Case 2. For the case 2, the idea is that the outcomes of each data points will already be known, i.e., it is already known if the record comes from crash prone dataset or normal traffic condition dataset. So basically here that information will be entered as evidence and adaptation algorithm will be applied.

CHAPTER 4

RESULTS & ANALYSIS

4.1 Introduction:

This chapter focuses on the detailed description of real-time crash prediction model building as well as the analytical part of the thesis. An elaborated illustration of the various techniques and algorithms used for the model building along with the different cases to consider is given in the previous chapter titled “Methodology”. The contents of this chapter aims to further extend the understandings from previous one and to guide through the process of analysis. The necessary steps involved in the analytical part, the procedures for data preparation will be demonstrated in various sub sections. Also a preliminary analysis of crash data will be incorporated to have a better understanding of the nature of the study area. In this study, HUGIN Expert, a commercially available BBN software tool for model building will be used. The working steps of the HUGIN Expert will be outlined in this chapter. The outcome of the analysis will be discussed and the results from various models will be compared. The evaluation of the model outcomes and the different nature of the models will also be given priority. A real-time crash prediction model should have a substantial level of accuracy for the model to be successfully able to predict crash events. A study on the accuracy of the models will be added in this chapter. Finally, comments on the performance of the real time crash prediction models and the level of achievement of the thesis objectives will be discussed.

4.2 Preliminary Analysis of crash data:

As it was stated before, a total of 230 crashes occurred in the study area Shibuya 3 inbound route through the months of March to August, 2014. The crash data obtained for the study has been primarily sorted out to run some analysis on the basic crash pattern, temporal and spatial variations of the crashes etc. A sample of the coded crash data is provided below in table 4.1 that is used for this study:

Figure 4.1: Crash Data Obtained for the study.

| Date | Time | Day of week | Weather code | Road surface code | Route number | Direction | kilo post |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6/1/2014 | 11:27 AM | Sunday | 01 | 01 | 003 | 01 | 6.00 |
| 6/1/2014 | 1:41 PM | Sunday | 01 | 01 | 003 | 01 | 11.70 |
| 6/2/2014 | 5:44 AM | Monday | 01 | 01 | 003 | 01 | 0.40 |
| 6/2/2014 | 8:42 AM | Monday | 01 | 01 | 003 | 01 | 9.90 |
| 6/4/2014 | 4:31 PM | Wednesday | 01 | 01 | 003 | 01 | 11.30 |
| 6/4/2014 | 5:48 PM | Wednesday | 01 | 01 | 003 | 02 | 1.00 |
| 6/6/2014 | 5:13 PM | Friday | 03 | 02 | 003 | 01 | 3.00 |
| 6/7/2014 | 1:12 AM | Saturday | 03 | 02 | 003 | 01 | 5.80 |
| 6/7/2014 | 10:46 AM | Saturday | 03 | 02 | 003 | 01 | 3.60 |
| 6/7/2014 | 9:25 PM | Saturday | 03 | 02 | 003 | 01 | 0.00 |
| 6/8/2014 | 10:53 PM | Sunday | 01 | 01 | 003 | 01 | 5.80 |
| 6/10/2014 | 2:28 AM | Tuesday | 02 | 02 | 003 | 01 | 0.10 |
| 6/10/2014 | 4:15 PM | Tuesday | 01 | 01 | 003 | 01 | 11.30 |
| 6/11/2014 | 5:52 AM | Wednesday | 02 | 01 | 003 | 01 | 1.70 |
| 6/12/2014 | 9:48 AM | Thursday | 03 | 02 | 003 | 01 | 11.30 |
| 6/12/2014 | 10:05 PM | Thursday | 02 | 01 | 003 | 01 | 6.80 |
| 6/13/2014 | 9:01 AM | Friday | 01 | 01 | 003 | 01 | 11.30 |
| 6/13/2014 | 3:22 PM | Friday | 01 | 01 | 003 | 01 | 11.00 |
| 6/13/2014 | 4:15 PM | Friday | 01 | 01 | 003 | 01 | 11.50 |
| 6/14/2014 | 6:25 AM | Saturday | 01 | 01 | 003 | 01 | 11.30 |
| 6/14/2014 | 10:34 AM | Saturday | 01 | 01 | 003 | 02 |  |
| 6/14/2014 | 11:32 AM | Saturday | 01 | 01 | 003 | 01 | 3.90 |
| 6/14/2014 | 4:25 PM | Saturday | 01 | 01 | 003 | 01 | 9.90 |

A comprehensive taxonomy is also included in the data sheet for complete understanding.

From a simple distribution of crash types spreading over out study area, it can be seen that the most dominant type of crash is rear end collision. Fifty seven percent of the crashes fall under this criteria. The second most prominent crash incident is vehicle contact. Thirty three percent of the crashes occur due to vehicle contact while facility contact takes up seven percent and falling object contact contributes three percent to total crash incidents. A very small percentage of crashes, 0.43 percent each, occur due to load collapse and roll over.

The distribution is illustrated below with the figure 4.1:

Figure 4.1: Distribution of crash types

Another analysis was carried out to find the most crash prone month within the time period of the study. The analysis reveals that March is the month with most number of crashes in the study area with 54 crashes occurring in this month. The month of August comes out with the second most crashes, a tally of 46 crashes. May is the month with lowest crashes with 28 crashes. Another figure 4.2 is provided here to show the monthly distribution of crashes throughout the study area:

Figure 4.2: Monthly Distribution of Crashes

The study also provides with a weekly analysis of crashes. As it happens Thursdays, Fridays and Saturdays of different months are the days with most number of crashes and Fridays occupy the peak value with 46 crashes occurring on Fridays throughout the study period. Figure 4.3 represents the weekly distribution:

Figure 4.3: Weekly Distribution of Crashes

From the hourly distribution of the crashes, it is seen that the crash occurrences start to gradually increase from 7 am in the morning. The most number of crashes occur in the 11 am to 12 pm time range. Then it slowly decreases but increases again in the 4 pm to 5 pm time interval. After that the crash frequency decreases. Figure 4.4 illustrates the hourly distribution:

Figure 4.4: Hourly Distribution of Crashes

This analysis depicts a clear image of the temporal distribution of crashes. And most crash prone time of the day, as indicated from the analysis is from 11 AM to 12 PM.

A spatial analysis of the study area in relation with the crash incidents is also presented in this study. The various crash locations are identified from the positions of the crash events and the relative kilo post locations. Figure 4.5 represents the spatial variation of the crash events:

Figure 4.5: Spatial Variation of crashes

The analysis indicates that the highest number of crashes, 23, occur near the kilo post 11.3 located in the study area. Also a significant number of crashes, 52, occur within 0.5 kilo meter area of 11.3 to 11.8. So this can be referred as a Black Spot for crashes. Table 4.2 is presented here showing the number of crashes in different locations of the study area:

Table 4.2: Summarization of Spatial Distribution of Crashes:

| Road length(Km) | Number of crashes |
| --- | --- |
| 0-1.9 | 64 |
| 2-3.9 | 30 |
| 4-5.9 | 21 |
| 6-7.9 | 43 |
| 8-9.9 | 11 |
| 10-11.9 | 61 |

4.3 Data Preparation:

In this section, the whole process of data preparation will be discussed. For developing BBN models using HUGIN Expert, a data sheet is first required for running the analysis. The whole process of preparing the data sheet for HUGIN Expert input is described in the following sub sections.

4.3.1 Data Extraction:

The study is based on the development of the BBN models to build real-time crash prediction model. And the development of the BBN models require a data set on all the variables involved. The data on the variables are required to be extracted from the crash data base as well as traffic flow data base. First, all the crashes are given a specific “Crash ID”. It is done to simplify the data base building procedure.

After the selection of the detectors from which the data will be extracted, the specific detectors are identified corresponding to each Crash ID. For example, for determining the specific detectors required, the task is to simply identify the crash location relative to the kilo post marks. Then from the detector data base, the nearest detectors just upstream and just downstream is selected. These detectors will then be used to get matching crash and normal traffic data. A table 4.3 illustrating this process is given below:

Table 4.3: Identification of detectors in respect to Crash IDs

| Crash ID | Date | Time | Day of week | kilo post | D2D | D1D | D0 | D1U | D2U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 6/7/2014 | 9:25 PM | Saturday | 0.00 | 03-01-02 | 03-01-03 | 03-01-01 | - | - |
| 2 | 6/10/2014 | 2:28 AM | Tuesday | 0.10 | 03-01-05 | 03-01-04 | 03-01-02 | 03-01-01 | 03-01-03 |
| 3 | 6/29/2014 | 3:26 AM | Sunday | 0.10 | 03-01-05 | 03-01-04 | 03-01-02 | 03-01-01 | 03-01-03 |
| 4 | 3/21/2014 | 5:47 PM | Friday | 0.10 | 03-01-05 | 03-01-04 | 03-01-02 | 03-01-01 | 03-01-03 |
| 5 | 8/15/2014 | 8:01 AM | Friday | 0.10 | 03-01-05 | 03-01-04 | 03-01-02 | 03-01-01 | 03-01-03 |
| 6 | 8/18/2014 | 4:22 PM | Monday | 0.10 | 03-01-05 | 03-01-04 | 03-01-02 | 03-01-01 | 03-01-03 |
| 7 | 6/29/2014 | 11:06 AM | Sunday | 0.20 | 03-01-05 | 03-01-04 | 03-01-02 | 03-01-01 | 03-01-03 |

After the identification of necessary detectors to get data from, the extraction process of the crash data and matching normal traffic data are carried out. As stated earlier, aggregated data of 1 minute of cumulative vehicle counts, average speed and average occupancy data are extracted for crash occurrences. The data sheet is prepared with some codes given to each variables according to the position of detectors. After the extraction of the crash data, corresponding normal traffic data are collected to complete the data base. Table 4.4 represents a sample of the preliminary data sheet:

Table 4.4: A sample of the preliminary data sheet

| Crash ID | 1 | 1 | 1 | 2 | 2 | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| Date | 6/7/2014 | 7/9/2014 | 6/17/2014 | 6/10/2014 | 6/17/2014 | 5/6/2014 |
| time | 21:25 | 21:25 | 21:25 | 2:28 | 2:28 | 2:28 |
| kilo post | 0.00 | 0.00 | 0.00 | 0.10 | 0.10 | 0.10 |
| D1D_1_f_5 | 5 | 8 | 7 | 3 | 2 | 7 |
| D1D_1_f_4 | 11 | 5 | 6 | 3 | 9 | 5 |
| D1D_1_f_3 | 18 | 9 | 6 | 4 | 9 | 4 |
| D1D_1_f_2 | 4 | 11 | 7 | 7 | 11 | 11 |
| D1D_1_f_1 | 2 | 6 | 4 | 6 | 7 | 8 |
| D1D_1_s_5 | 75 | 75 | 65.6 | 69.2 | 85.7 | 72.4 |
| D1D_1_s_4 | 64.7 | 75 | 75 | 81.8 | 73 | 65.2 |
| D1D_1_s_3 | 60.7 | 66.7 | 75 | 66.7 | 63.8 | 75 |
| D1D_1_s_2 | 85.7 | 73.3 | 70 | 70 | 75 | 75 |
| D1D_1_s_1 | 75 | 69.2 | 85.7 | 75 | 84 | 75 |

From the table presented here it can be seen that crash data are color coded as red and two sets of corresponding normal traffic data(color coded as blue) from randomly chosen days(in which no crashes have taken place) are extracted and stored for further analysis.

4.3.2 Aggregating the data:

The variable data are collected from two different detectors for each lane. So for one point, two detector data are available. Now for the analysis, two detector data are aggregated to form the variables used in the analysis process. As it was mentioned earlier the speed and occupancy data are taken as average while the flow data is cumulated for two detectors.

4.3.3 Congestion Index calculation:

The theory on Congestion Index and its equation are explained in the previous chapter. The Greenshields Model is adopted to find the free flow speed in this thesis. From the Greenshields theory:

The classical equation:  K =  Kj -  Kj* VVf                                                                                          (4.1)

The regression line equation: K = 36.585631 - 0.389404V                                                           (4.2)

After that from regression analysis, the parameters of the equation are identified and the free flow speed, Vf is found to be 93.95 km/hour. Plugging in that value and the 1 min aggregated avg. speed in the following equation, CI is calculated:

Congestion Index (CI) = (Free Flow Speed - Speed)/Free Flow Speed

When CI>0 = 0; when CI<=0.

4.3.4 The Final Datasheet for HUGIN Expert:

For the model development, the outcome variable is selected as “Crash” and denoted as a binary variable. So “Crash” is denoted as “1” and “Non Crash” is denoted as “0” as binary variables. After that the variable data and CI is included and a complete data sheet for HUGIN Expert is created. All the models will be developed from this final data sheet. Table 4.5 is provided below to represent a part of the final data sheet:

Table 4.5: A part of the final data sheet

| Crash | U1_S_5 | U1_O_5 | U1_F_5 | U1_S_4 | U1_O_4 | U1_F_4 | U1_S_3 | U1_O_3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| "1" | 29.7 | 25.9 | 42 | 20.4 | 27.65 | 39 | 29.95 | 21.85 |
| "0" | 72.95 | 11.65 | 58 | 72.4 | 8.65 | 45 | 71.15 | 7.85 |
| "0" | 79.35 | 9.95 | 50 | 71.5 | 7.85 | 42 | 79.55 | 8.65 |
| "1" | 31.05 | 23 | 49 | 39.5 | 19.85 | 52 | 21.4 | 34.2 |
| "0" | 21.55 | 27.4 | 43 | 39.75 | 20 | 52 | 49.65 | 16.45 |
| "0" | 8.15 | 53.4 | 20 | 4.2 | 57 | 13 | 18.1 | 27.7 |
| "1" | 79.05 | 5.8 | 33 | 77.45 | 4.7 | 26 | 80.85 | 4.25 |
| "0" | 80.7 | 4 | 15 | 70.3 | 9.45 | 40 | 79.6 | 4.7 |
| "0" | 78.3 | 6.35 | 31 | 80.6 | 4.9 | 23 | 70.6 | 4.05 |
| "1" | 55.9 | 15.4 | 57 | 28.8 | 28.3 | 48 | 27.2 | 25.4 |
| "0" | 71.9 | 7.95 | 37 | 73.4 | 7.85 | 36 | 67.2 | 13.2 |
| "0" | 74.75 | 10.3 | 47 | 75.95 | 9 | 44 | 75.2 | 9.15 |
| "1" | 65.2 | 10.6 | 54 | 76.5 | 6.6 | 34 | 76.4 | 9.1 |
| "0" | 67.75 | 11.2 | 51 | 75 | 6.9 | 40 | 64.35 | 11.95 |
| "0" | 71.65 | 8.1 | 39 | 70.55 | 11.7 | 58 | 68.5 | 11.3 |

4.4 Model Development:

This section deals with the model building process which is accomplished by HUGIN Expert. In the following subsections, the total process will be explained as to how the model is built from the scratch.

4.4.1 Variable Selection:

For the construction of the real-time crash prediction model, the variables to be used have already been identified in the previous chapter. However, in this study, 5 BBN models will be developed. The selected variables will be used but for different minutes leading up to the crash events. So 5 BBN models will use the variables for each of the minute prior to crash. These are labeled as decision variables. And all the models will have “Crash” as the same outcome variable. A comprehensive table 4.6 for the variables used in different models for each of the minutes is illustrated here:

| Model Name | Built for the minute prior to Crash | Decision Variables | Outcome Variable |
| --- | --- | --- | --- |
| Model 1 | 1 | D1_O_1, D1_S_1, D1_F_1, U1_F_1, U1_S_1, U1_O_1 , CI_U_1,CI_D_1 | Crash |
| Model 2 | 2 | D1_O_2, D1_S_2, D1_F_2, U1_F_2, U1_S_2, U1_O_2 , CI_U_2,CI_D_2 | Crash |
| Model 3 | 3 | D1_O_3, D1_S_3, D1_F_3, U1_F_3, U1_S_3, U1_O_3 , CI_U_3,CI_D_3 | Crash |
| Model 4 | 4 | D1_O_4, D1_S_4, D1_F_4, U1_F_4, U1_S_4, U1_O_4 , CI_U_4,CI_D_4 | Crash |
| Model 5 | 5 | D1_O_5, D1_S_5, D1_F_5, U1_F_5, U1_S_5, U1_O_5 , CI_U_5,CI_D_5 | Crash |

Table 4.6: Summarization of selected variables

The corresponding values of these variables will be used to find out their interaction among themselves and the influences of these variables on crash event.

4.4.2 Model Construction with HUGIN Expert:

The model building process in HUGIN Expert, the software used to build and evaluate the BBN models, involves a series of complex actions. The complete data sheet developed specifically for the software is first introduced to the software. After that various tasks are performed to construct the BBN models. The step-by-step process in the HUGIN Expert is described below:

- Discretization:

As stated in the previous sub section, the different decision variables and the outcome variable are the main components for the models. The decision variables are numeric in nature. But the outcome variable “crash” is set in binary. So it poses a problem as the variable types do not match. To overcome this problem, discretization is performed to convert the numeric variables into discrete variables in user defined categories. For this study, each of the numeric variable is discretized and divided into five uniform categories. This process is done by a built-in tool provided by the software.

- Structure Learning:

After discretizing the variables, the next step is to learn the structure. It is discussed earlier that NPC algorithm, as a structure learning tool, is going to be used in this study. For learning the structure a constraint is set:

- The outcome variable “Crash” can be parent to none.

Afterwards, the HUGIN Expert will be given command to run the NPC algorithm. But before proceeding to this part, a level of significance has to be chosen. In this study, level of significance is set at 0.05. This level of significance is set as default in the software.

After this a structure will be provided by the software.

- Batch Learning:

The next step would be the batch learning process. The batch learning is going to be performed with the help of Expectation-Maximization algorithm. After applying the EM algorithm, the final BBN models are delivered as outputs by the software.

4.4.3 BBN Models:

By following the procedure explained in the previous sub section, 5 BBN models are developed for each of the 5 minutes leading up to the crash event. For building the models, 200 of crash data out of 230 have been used along with corresponding 400 normal traffic data. The rest 30 crash data and 60 normal condition traffic data are kept to be incorporated into the model later to evaluate the performance of the models. In this sub section, all the 5 BBN models will be demonstrated with their final structure as well as the mathematical equations. Also different interactions among the decision variables and the outcome variable will be briefly discussed. The 5 BBN models are:

- MODEL 1:

A total of 9 variables (1 minute prior to the crash) are selected to be introduced into the software and after going through the process and the obtained structure of the BBN for MODEL 1 is shown in figure 4.6:

Figure 4.6: BBN MODEL 1.

From the structure of the model, it is seen that “crash” is directly influenced by three   variables namely U1_F_1 (flow in upstream), D1_O_1 (occupancy in downstream) and D1_F_1 (flow in downstream).

The mathematical equation for the model can be written as:

P(Crash,D1_O_1,D1_S_1,D1_F_1,U1_F_1,U1_S_1,U1_O_1,CI_U_1,CI_D_1)= P(U1_O_1).P(D1_O_1).P(D1_S_1).P(CI_U_1|U1_O_1,U1_F_1).P(D1_F_1|D1_O_1,D

1_S_1).P(U1_F_1|U1_O_1,D1_F_1).P(U1_S_1|D1_O_1,CI_U_1).P(CI_D_1|D1_S_1,D1_F_1).P(Crash| D1_O_1,D1_F_1,U1_F_1)                                                               (4.3)

- MODEL 2:

Here variables for 2 minutes prior to the crash are utilized. Figure 4.7 shows the BBN MODEL 2:

Figure 4.7: BBN MODEL 2

So this model can be interpreted as crash is directly influenced by two variables: D1_S_2 (downstream avg. speed) and CI_D_2 (downstream congestion index).

The mathematical equation for the model is: P(Crash,D1_O_2,D1_S_2,D1_F_2,U1_F_2,U1_S_2,U1_O_2,CI_U_2,CI_D_2)=                           P(U1_O_2|U1_F_2,U1_S_2).P(D1_O_2|D1_F_2,D1_S_2).P(D1_S_2|CI_D_2).P(CI_U_2).P(D1_F_2|U1_F_2).P(U1_F_2|CI_U_2).P(U1_S_2|CI_U_2).P(CI_D_2).P(Crash|D1_S_2,CI_D_2)                                                                                                               (4.4)

- MODEL 3:

The model 3 was built for just 3 minutes before the crash incident and the constructed model is presented in figure 4.8:

Figure 4.8: BBN MODEL 3

As it can be seen from the structure, for just 3 minutes prior to the crash, no influence on crash event from the variables is found. So from the model obtained for minute 3 is not giving any prediction on crash event.

- MODEL 4:

Again, MODEL 4 was constructed with the variables for 4 minutes prior to crash. The final structure of the model is given by the figure 4.9:

Figure 4.9: BBN MODEL 4

From this model, it is clearly seen that crash is directly influenced only by U1_S_4 (upstream avg. speed).

The mathematical expression for this model can be stated as:

P(Crash,D1_O_4,D1_S_4,D1_F_4,U1_F_4,U1_S_4,U1_O_4,CI_U_4,CI_D_4)=                           P(U1_O_4|U1_F_4).P(D1_O_4|D1_S_4).P(D1_S_4|CI_D_4).P(CI_U_4|U1_S_4,U1_O_4,U1_F_4).P(D1_F_4|CI_D_4).P(U1_F_4|D1_F_4).P(U1_S_4|U1_O_2,U1_F_4).P(CI_D_4).P(Crash|U1_S_4)                                                                                                      (4.5)

- MODEL 5:

Just in the same procedure as before, the MODEL 5 is developed by using the variables just 5 minutes prior to crash. The developed model can be illustrated below in figure 4.10:

Figure 4.10: BBN MODEL 5

From the illustration, it is observed that just like MODEL 3, the model developed with variables of 5 minutes prior to crash do not have any influence on crash event.

So, basically the above subsections provide the general concept of the model developing technique as well as the illustration and mathematical expressions of the developed BBN models. Moreover, the working steps of HUGIN Expert software are also explained briefly to provide a better understanding of the work flow of the study. Now further analysis of the models and the acquired results will be discussed in the rest of this chapter.

4.5 Results of the Models:

After the model has been built, the different results in terms of probability of crash and non-crash events are found. The summary of the result is shown in the table 4.7 below:

Table 4.7: Summary of the results

| Models | Probability of Crash | Probability of Non-Crash |
| --- | --- | --- |
| Model 1 | 38.45 | 61.55 |
| Model 2 | 33.33 | 66.67 |
| Model 3 | 33.33 | 66.67 |

From the table, it can be seen that crash probability for model 1, just one minute prior to the crash, is the highest with a value of 38.45 percent. And crash probability are found to be 33.33 percent for the rest of the two models. Now these values will be called “Crash Threshold” values.

The performance evaluation of the BBN models will be done comparing the crash predictions of the updated models with these “Crash Threshold” values.

4.6 Evaluation:

For the evaluation of the prepared BBN models, two cases have been selected to consider. For the Case 1, 30 crash data and 60 randomly chosen normal condition traffic data have been entered into the model. Here Bayesian outputs of “Crash” and “Non-Crash” are found. This results will be compared with Case 2.

After that Case 2 is considered. In this part of the evaluation, the 30 crash data and 60 non crash data are alternatively entered into the models. The new models provide new probability of crash and non-crash events. Table 4.8 here shows the outcomes:

| Crash | P(crash_4) | P(crash_2) | P(crash_1) |
| --- | --- | --- | --- |
| "1" | 42.95 | 20 | 19.12 |
| "0" | 20.24 | 17 | 14.63 |
| "0" | 20.24 | 17 | 13.33 |
| "1" | 44.46 | 63.58 | 58.82 |
| "0" | 44.46 | 63.58 | 33.33 |
| "0" | 42.95 | 52.4 | 0 |
| "1" | 20.24 | 20 | 29.12 |
| "0" | 20.24 | 17 | 24.12 |
| "0" | 20.24 | 17 | 29.12 |
| "1" | 44.46 | 42.27 | 56 |
| "0" | 20.24 | 40 | 13.33 |
| "0" | 20.24 | 40 | 50 |
| "1" | 20.24 | 40 | 14.63 |
| "0" | 20.24 | 20 | 36 |
| "0" | 20.24 | 20 | 14.63 |

Table 4.8: Results from Case 2

Then the values are compared with the “Crash Threshold” values from the original BBN models. Two sets of results are then obtained. From there, the crash and non-crash outcomes from Bayesian outputs and the actual events that took place in real life are compared to find the accuracy of the models. For determining the accuracy and false alarm rates, matrices are developed with the following components:

- The actual events that took place in real life and,

- The outputs of BBN models.

The results of this evaluations are:

- MODEL 1: From the results, it is seen that the number of crash events as identified by the BBN model 1 matches on 13 occasions with actual cases where crashes occurred. And number of non-crash events matches with actual scenarios are 50. Here table 4.9 presents the results:

| MODEL 1 |  | Bayesian Model Outputs |  |  |
| --- | --- | --- | --- | --- |
|  |  | CRASH_1 | NON CRASH_1 |  |
| Actual | CRASH_1 | 13 | 17 | 30 |
|  | NON CRASH_1 | 40 | 50 | 60 |

Table 4.9: Evaluation of MODEL 1

- MODEL 2: In case of model 2, 21 crash events predicted by BBN model 2 matches with actual events where as 32 non-crash events matches with actual incidents. Table 4.10 shows the results:

Table 4.10: Evaluation of MODEL 2

| MODEL 2 |  | Bayesian Model Outputs |  |  |
| --- | --- | --- | --- | --- |
|  |  | CRASH_2 | NON CRASH_2 |  |
| Actual | CRASH_2 | 21 | 9 | 30 |
|  | NON CRASH_2 | 28 | 32 | 60 |

- MODEL 4: Same procedure is followed to evaluate the performance of Model 4. Here it is observed that 8 of the BBN model 4 outputs matches actual crash incidents that took place. And 46 of the non-crash events predicted by the BBN model corresponds with actual events. Table 4.11 illustrates the results here:

Table 4.11: Evaluation of MODEL 4

| MODEL 4 |  | Bayesian Model Outputs |  |  |
| --- | --- | --- | --- | --- |
|  |  | CRASH_4 | NON CRASH_4 |  |
| Actual | CRASH_4 | 8 | 22 | 30 |
|  | NON CRASH_4 | 14 | 46 | 60 |

For a better evaluation of the models, accuracy of crash prediction, accuracy of non-crash prediction, overall accuracy of the models as well as the false alarm rates of the BBN models are calculated. The summary of the results are shown in the table 4.12:

Table 4.12: Summary of Evaluation:

| BBN MODELS | Overall accuracy | Crash accuracy | Non crash accuracy | False Alarm |
| --- | --- | --- | --- | --- |
| MODEL 1 | 70% | 43% | 83% | 63% |
| MODEL 2 | 58% | 70% | 53% | 41% |
| MODEL 4 | 60% | 26% | 76% | 40% |

From the table, it is observed that the overall accuracy of the models are quite satisfactory. The Model 1 has an overall accuracy of 70%, with crash accuracy of 43%. But the false alarm rate is 63% for this model.

Model 2 has an accuracy of 58% overall with a crash accuracy of 70%. The non-crash accuracy is also quite satisfactory with a value of 53%. The false alarm rate is 41% for this model.

In case of Model 4, the overall accuracy is 60%. But it has a low crash accuracy with only 26%. The non-crash accuracy is as high as 76% and false alarm rate is observed to be 40%.

From the evaluation of the models, the results seem quite satisfactory. The percentages of predicting crash events successfully is significant. Also the overall accuracy of the models are impressive. The false alarm rates may be a little higher in the models. However, it is expected that when the models will be implemented and when they will get substantial amount of exposure to normal traffic condition, it will be able to reduce false alarm rates.

4.7 Classical problems with real-time crash prediction models and performance of Bayesian Belief Nets:

Over the years, many approaches have been taken to perfect the concept of real-time crash prediction models. In Chapter 2, those approaches and their significances are discussed thoroughly. This study implemented Bayesian Belief Nets as a platform to develop real-time crash prediction models. The classical problems associated with previous models have been addressed in this study to improve the results. Such as-

The need to have sufficient data- One of the major hindrances to this field of study is to have sufficient number of crash data with matching sensor data. In this study, a very small sample size has been used. The BBN models allowed the use of small sample to effectively develop real-time crash prediction models. Thus the problems associated with the need of large data samples can be eliminated.

Difficulty in acquiring data from different sensors- As there are different types of detectors can be present on the network collecting different type of traffic variable data, the data acquisition for relevant variables can become a problem. Also the addition of new variables can rise the need to build the model again. But BBN models have the inherent capability to incorporate new variables into the existing model without needing to build it from the scratch.

Performance- A very important feature for a real-time crash prediction model is the performance of the model, especially the accuracy and false alarm rates. The study has achieved relatively acceptable level of accuracy and fairly low false alarm rates.

So the study results and findings suggest that most of the major problems can be resolved by the implementation of BBN models as real-time crash prediction models.

4.8 Summary:

The analysis part of the thesis is very important to achieve the desired results specifically mentioned in the objectives. In this chapter, the whole analysis process is carefully documented. The data preparation and preliminary analysis are the first part of the analysis. And then, the step by step process in the HUGIN Expert software is laid out in detail. After a careful analysis, 3 out of 5 BBN models show how the different variables influence the crash events, and the other 2 BBN models did not show any definite influence of the variables on the crash event. The overall accuracy and crash accuracy of the BBN models along with false alarm rates have been determined to evaluate the performance of the models. A close observation reveals that the prepared real-time crash prediction models are fairly accurate with a significant accuracy in spotting crash events and non-crash events.

CHAPTER 5

CONCLUTIONS

5.1 Introduction:

The concept of real-time crash prediction models may still be relatively new but the need to have crash prediction models that can work in real-time with precision is now rapidly growing. The idea that crash events can be predicted and proper intervention can even prevent such events has been attracting massive attention from researchers all over the world. In addition to contributing to the existing researches, this thesis tries to develop a model that can be reliable in respect of performance and the accuracy of the models. This chapter provides a general idea about the results of the thesis with respect to the specific objectives set in the first chapter. The limitations of the constructed real-time crash prediction models is discussed. Also the future scope of the study is pointed out in this chapter of the thesis.

  - Objective Based Conclusion:

This subsection of the thesis addresses the objectives specified in the first chapter of the thesis and discusses the level of accomplishment of the stated objectives.

- Develop real-time crash prediction models with BBN:

One of the objectives for the study is to develop real-time crash prediction models with BBN. For the major part of the thesis, this objective has been given priority. The previous chapters discussed about the modelling techniques used to accomplice this objective. And in this study, 5 BBN models are constructed. The models reflect the significant requirements of real-time crash prediction models. The different interactions among the different variables of the system along with their influence on the crash phenomena have been identified. Among the models, 3 BBN models showed how the variables interact and influence the crash phenomena. From the constructed models, the significant variables can be identified. Along with the traffic flow variables in both upstream and downstream detectors-1 minute aggregated average speed, 1 minute aggregated average occupancy, cumulative 1 minute vehicle count; another variable namely “Congestion Index” are found to be significantly influencing the crash phenomena. So the models can be implemented given that the selected significant variables data can be collected from the proposed expressway. There are some other objectives that have been achieved in the process such as the application of NPC algorithm in structure learning of the BBN models. Also the use of Expectation-Maximization (EM) algorithm for parameter estimation is reflected in the study.

- To evaluate the performance of the developed models:

The second objective of the study is to evaluate the constructed real-time crash prediction models on the basis of performance and accuracy. Chapter 4 gives a comprehensive detail on the accuracy and the performance of the developed models. The overall accuracy of the models found to be quite satisfactory. The false alarm rates are also estimated as a part of the evaluation process. The results suggest that the study achieved an acceptable level of outcome and the generated results can have positive impacts in reducing crash phenomena depending on their transferability to real life scenarios. From the study, for BBN MODEL 2, the overall accuracy is found to be 58% with crash accuracy of 70% and false alarm rate of 41%. When compared with the other models, MODEL 2 built with variable data of just 2 minutes before the crash gives the most satisfactory result in predicting crash events successfully. However, as the model is built with variable data just 2 minutes from crash event, it can significantly reduce the chances of applying intervention just in time. Hence the BBN MODEL 4 having an overall accuracy of 60%, crash accuracy of 26% and false alarm rate of 40% can be definitely selected as the most viable model as it is built with variable data of just 4 minutes prior to the crash event. So there can be ample time to intervene before the crash phenomena. So in terms of performance evaluation, the constructed BBN models show great promises to be successfully implemented.

5.3 Limitations and Future Scope:

This subsection concludes the thesis by stating the limitations of the study. Also, further advancement that can be ventured from this study is presented as future scope in this subsection.

- The study is conducted only in Shibuya 3 inbound route of Tokyo Metropolitan Expressway, Tokyo, Japan. The data collected is also small as sample size for the construction of the models. For further study, increase in sample size and addition of new routes can be considered.

- The transferability of the model is also a problem because the study area used is equipped with modern sensors and densely spaced loop detectors that collect traffic data round the clock. So difficulties may arise in implementing the study in different expressways. For the implementation of the study, a practical study from the concerned area is required to have a definite conclusion in deciding if the models from this study can be implemented.

- The study includes only the basic freeway segments. The effects of ramps are not considered in building the models. The futures scope from this respect is to consider the whole expressway without dividing it into only basic freeway segments.

- The availability of the matching crash and normal traffic data can also pose a problem while implementing the study in other expressways.

- The study emphasizes on predicting the crash phenomena, but the prevention methods or intervention techniques are not discussed in this study. These techniques and methods can be included in future works to achieve more thorough impact and to increase the advantages of the models in case of implementation.

REFERENCES

Abbess, C., Jarett, D. and Wright, C.C., 1981. Accidents at Blackspots: Estimating the Effectiveness of Remedial Treatment, With Special Reference to the Regression-to-Mean Effect. Traffic Engineering and Control, Vol. 22, No. 10, pp. 535-542.

Abdel-Aty, M., Abdalla, F., 2004. Linking roadway geometrics and real-time traffic characteristics to model daytime freeway crashes using generalized extreme equations for correlated data. Transportation Research Record 1897, 106–115.

Abedl-Aty, M., A. Pande, A. Das and W. J. Knibbe. 2008. Assessing safety on Dutch Transportation Research Record: Journal of the Transportation Research Board, No. 2083, Transportation Research Board of National Academics, Washington, D.C., 2008, pp. 153-161.

Abdel-Aty, M., Dilmore, J., Dhindsa, A., 2006. Evaluation of variable speed limits for real-time freeway safety improvement. Accident Analysis and Prevention 38 (2), 335–345.

Abdel-Aty, M., Pande, A., 2004. Classification of real-time traffic speed patterns to predict crashes on freeways. In: Proceedings of the 83rd Annual Meeting of Transportation Research Board, Washington, DC.

Abdel-Aty M. and Pande A., 2005. Identifying crash propensity using specific traffic speed conditions, Journal of Safety Research, Vol. 36 No.1, pp. 97-108.

Abdel-Aty, M. and Pande, A., 2006. ATMS Implementation System for Identifying Traffic Conditions Leading to Potential Crashes. IEEE Transactions on Intelligent Transportation Systems, Vol. 7, No. 1. pp. pp. 78-91.

Abdel-Aty, M., Pemmanaboina, R., Hsia, L., 2006. Assessing crash occurrence on urban freeways by applying a system of interrelated equations. Transportation Research Record 1953, 1–9.

Abdel-Aty, M., Pande, A., Lee, C., Gayah, V. And Santos, C. D., 2007. Crash Risk Assessment Using Intelligent Transportation Systems Data and Real-Time Intervention Strategies to Improve Safety on Freeways. Journal of Intelligent Transportation Systems, Vol. 11, No. 3, pp. 107-120.

Abdel-Aty, M., Uddin, N., Pande, A., Abdalla, F. M., & Hsia, L., 2004. Predicting freeway crashes from loop detector data by matched case–control logistic regression. Transportation Research Record: Journal of the Transportation Research Board, 1897, pp. 88–95.

Abdel-Aty, M., Pande, A., Lee, C., Gayah, V. And Santos, C. D., 2007. Crash Risk Assessment Using Intelligent Transportation Systems Data and Real-Time Intervention Strategies to Improve Safety on Freeways. Journal of Intelligent Transportation Systems, Vol. 11, No. 3, pp. 107-120.

Abdel-Aty, M., Pande, A., Das, A. and Knibbe, W. J., 2008. Assessing Safety on Dutch Freeways with Data from Infrastructural-based Intelligent Transportation Systems. Transportation Research Record: Journal of the Transportation Research Board, No. 2083, TRB, National Research Council, Washington, D. C., pp. 153-161.

Abdel-Aty, M., and V. Gayah, 2010. Real-Time Crash Risk Reduction on Freeways Using Coordinated and Uncoordinated Ramp Metering Approaches. Journal of Transportation Engineering, Vol. 136, 2010, pp. 410-423.

Ahmed, M.M., Mohamed A., Jaeyoung L., Rongjie Y., 2014. Real-time assessment of fog-related crashes using airport weather data: A feasibility analysis. Accident Analysis and Prevention 72 (2014) pp. 309-317.

Bossche, F. V., Wets, G., and Brijs, T., 2006. Predicting Crashes using Calendar Data. 85 th TRB Annual Meeting, Washington DC., USA.

Brodsky, H., Hakkert, A.S., 1988. Risk of a Road Accident in Rainy Weather. Journal of Accident Analysis and Prevention. Vol. 20, No. 2, pp. 161–176.

Caliendo, C., Guida, M., Parisi, A., 2007. A crash-prediction model for multilane roads. Accident Analysis and Prevention 39 (4), pp. 657–670.

Chapman, R.A. (1973). The concept of exposure. Journal of Accident Analysis and Prevention. Vol. 49 5, No. 2, pp. 95–110.

Ceder, A., 1980. Relationships between Road Accidents and Hourly Traffic Flow: II. Probabilistic Approach, Technion-Israel Institute of Technology, Transportation Research Institute, Haifa, Israel. Publication No. 80-29.

Cedar, A. and Livneh, M., 1982. Relationship between Road Accidents and Hourly Traffic Flow. Journal of Accident Analysis and Prevention, Vol. 14, pp. 19-44.

Ciro Caliendo, Maria Luisa De Guglielmo , Maurizio Guida, 2013. A crash-prediction model for road tunnels. Accident Analysis and Prevention 55 (2013) pp. 107-115.

Dias, C., Miska, M., Kuwahara, M., Warita, H., 2009. Relationship between congestion and traffic accidents on expressways: an investigation with Bayesian belief networks. In: Proceedings of 40th Annual Meeting of Infrastructure Planning (JSCE), Japan.

Donnell, E. T. and J. M. Mason. Methodology to Develop Median Barrier Crash Warrant Criteria. Journal of Transportation Engineering, American Society of Civil Engineers, Vol. 132, No. 4, April 2006, pp. 269-281.

Dingus, T. A., Hetrick, S. and Mollenhauer, M., 1999. Empirical Methods in Support of Crash Avoidance Model Building and Benefits Estimation. Journal of Intelligent Transportation Systems, Vol. 5 No. 2, pp. 93-125.

Eric T. Donnell, John M. Mason Jr., 2006. Predicting the frequency of median barrier crashes on Pennsylvania interstate highways, Accident Analysis & Prevention, Volume 38, Issue 3, May 2006, pp. 590-599.

Fridstrom, L., Ifver, J., Ingebrigtsen, S., Kulmala, S.R., Thomsen, L.K., 1995. Measuring the contribution of randomness, exposure, weather, and daylight to the variation in road accident counts. Accident Analysis and Prevention 27 (1), 1–20.

Golob, T. F., Recker, W. W., and Alvarez, V. M., 2004. Freeway Safety as a Function of Traffic Flow. Journal of Accident Analysis and Prevention, Vol. 36, No. 6, 933-946.

Golob, T. F., Recker, W. W., and Alvarez, V. M., 2004. Freeway Safety as a Function of Traffic Flow. Journal of Accident Analysis and Prevention, Vol. 36, No. 6, pp. 933-946.

Gwynn, D.W., 1967. Relationship of Accident Rates and Accident Involvement with Hourly Volumes. Traffic Quarterly, Vol. 21, pp. 407-418.

Hasan, Hany M., M. Abdel-Aty, 2013. Predicting reduced visibility related crashes on freeways using real-time traffic flow data. Journal of Safety Research 45 (2013), pp. 29-36.

Hauer, E., 2001. Over dispersion in Modelling Accidents on Road Sections and in Empirical Bayes Estimation. Journal of Accident Analysis and Prevention. Vol. 33, 799–808.

Hellinga, B., and M. Mandelzys, 2011. Impact of driver compliance on the safety and Operational impacts of freeway variable speed limit systems. Journal of Transportation Engineering, Vol. 137, 2011, pp.260-268.

Heydecker, B.G., J. Wu, 2001. Identification of Sites for Road Accident Remedial Work by Bayesian Statistical Methods: An Example of Uncertain Inference. Advances in Engineering Software, Vol. 32, pp. 859-869.

Hossain, M., Muromachi, Y., 2011. Development of a real-time crash prediction model for urban expressway. Journal of EASTS 8, pp. 2091–2107.

Hossain, M., and Y. Muromachi, 2012. A Bayesian Network based framework for real-time crash prediction on the basic freeway segments of urban expressways. Journal of Accident Analysis and Prevention, Vol. 45, 2012, pp. 373-381.

Hossain, M., and Y. Muromachi, 2013. Understanding crash mechanism on urban expressways using high-resolution traffic data. Journal of Accident Analysis and Prevention, Vol. 57, 2013, pp.17-29.

Jang, J.A., Choi, K., Cho, H., 2012. A fixed sensor-based intersection collision warning system in vulnerable line-of-sight and/or traffic-violation-prone environment. Intelligent Transportation System 13 (4), 1880–1890.

Jensen, F. V. and T.D. Nielsen. Bayesian Network and Decision Graphs. 2nd Edition, Springer, New York, USA, 2007.

Khan, S., Shanmugam, R., Hoeschen, B., 1999. Injury, fatal, and property damage accident models for highway corridors. Transportation Research Record 1665, 84–92.

Kjaerulff, U. B. and A. L. Bayesian Networks and Influence Diagrams: A guide to construction and analysis. Springer, New York, USA, 2008.

Lee, C., Szccomanno, F., Hellinga, B., 2002. Analysis of crash precursors on instrumented freeways. Transportation Research Record 1784, 1–8.

Lee, C., Hellinga, B., Saccomanno, F., 2003. Real-time crash prediction model for the application to crash prevention in freeway traffic. Transportation Research Record 1840, 67–77.

Lee. C., and M. Abdel-Aty., 2008. Testing effects of warning messages and variable speed limits on driver behavior using driving simulator. Transportation Research Record: Journal of the Transportation Research Board, No. 2069, Transportation Research Board of National Academics, Washington, D.C., 2008, pp.55-64.

Lord, D. (2000). The Prediction of Accidents on Digital Networks: Characteristics and Issues Related to the Application of Accident Prediction Models. Doctoral Dissertation, University of Toronto, Graduate Department of Civil Engineering. National Library of Canada, Ottawa, Canada.

Lord, D., S.P. Washington, and J.N. Ivan (2005) Poisson, Poisson-Gamma and Zero Inflated Regression Models of Motor Vehicle Crashes: Balancing Statistical Fit and Theory. Accident Analysis & Prevention. Vol. 37, No. 1, pp. 35-46.

Lord, D., and L.F. Miranda-Moreno (2006) Effects of Low Sample Mean Values and Small Sample Size on the Estimation of the Dispersion Parameter of Poisson gamma Models: A Bayesian Perspective. Working Paper to be submitted for publication. Department of Civil Engineering, Texas A&M University, College Station, TX.

Miaou, S. P., 1994. The Relationship between Truck Accidents and Geometric Design of Road Sections: Poisson versus Negative Binomial Regressions. Journal of Accident Analysis and Prevention, Vol. 26, No. 4, pp. 471−482.

Miaou, S.-P., Song, J.J., 2005. Bayesian ranking of sites for engineering safety improvements: decision parameter, treatability concept, statistical criterion and spatial dependence. Accident Analysis and Prevention 37 (4), 699–720.

Ogden, B. D., 2007. Railway-Highway Grade Crossing Handbook. Revised 2nd Edition, U. S. Department of Transportation, Federal Highway Administration, Washington D.C.

Oh, C., Oh, J. S. and Ritchie, S.G., 2000. Real-Time Estimation of Freeway Accident Likelihood. Institute of Transportation Studies, Universality of California, Irvine, CA. Working Paper ID: UCIITS-TS-WP-00-8.

Oh, C., Oh, J., Ritchie, S., Chang, M., 2001. Real-time estimation of freeway accident likelihood. In: Proceedings of the 80th Annual Meeting of Transportation Research Board, Washington, DC.

Oh, J., Oh, C., Ritchie, S., Chang, M., 2005a. Real time estimation of accident likelihood for safety enhancement. ASCE Journal of Transportation Engineering 131 (5), 358–363.

Oh, C., Oh, J., Ritchie, S., Chang, M., 2005b. Real time hazardous traffic condition warning system: framework and evaluation. IEEE Transactions on Intelligent Transportation Systems 6 (3), 265–272.

Pande, A., M. Abdel-Aty, 2005. A freeway safety strategy for advanced proactive traffic management. Journal of Intelligent Transportation Systems 9 (3), 145–158.

Pande, A. and Abdel-Aty, M., 2006a. Comprehensive Analysis of the Relationship between Real-time Traffic Surveillance Data and Rear-end Crashes on Freeways. In Transportation Research Record: Journal of the Transportation Research Board, No. 1953, Transportation Research Board of the National Academics, Washington, D.C., pp. 31-40.

Pande, A. and Abdel-Aty, M., 2006b. Assessment of Freeway Traffic Parameters Leading to Lane change Related Collisions. Journal of Accident Analysis and Prevention, Vol. 38, pp. 936-948.

Pande, A. and Abdel-Aty, M., 2007. Multiple-Model Framework for Assessment of Real-Time Crash Risk. In Transportation Research Record: Journal of the Transportation Research Board, No. 2019, Transportation Research Board of the National Academics, Washington, D.C., pp. 99-107.

Pande, A., M. Abdel-Aty, 2009. A novel approach for analyzing severe crash patterns on multi lane highways. Accident Analysis and Prevention 41 (2009) pp. 985–994.

Persaud, B., Lyon, C., Nguyen, T., 1999. Empirical Bayes Procedure for Ranking Sites for Safety Investigation by Potential for Safety Improvement. In Transportation Research Record: Journal of the Transportation Research Board, No. 1665, Transportation Research Board of the National Academics, Washington, D.C., pp. 7–12.

Pei, X., S.C. Wong, and N.N. Sze, 2012. The roles of exposure and speed in road safety analysis. Accident Analysis and Prevention 48(2012) 464-471.

Qin, X., Ivan, J.N., Ravishanker, N., Liu, J., (2005). Hierarchical Bayesian Estimation of Safety Performance Functions For Two-Lane Highways using Markov Chain Monte Carlo Modeling. Journal of Transportation Engineering, Vol. 131, No. 5, pp. 345–351.

Rongjie Yu, M. Abdel-Aty, 2013. Investigating the different characteristics of weekday and weekend crashes. Journal of safety research 46 (2013) pp. 91-97.

Roshandel S., Zuduo Z., Simon W., 2015. Impact of real-time traffic characteristics on freeway crash occurrence: Systematic review and meta-analysis. Accident Analysis and Prevention 79 (2015) pp. 198-211.

Schalkwyk, I., Mitra, S., and Washington, S., 2006. Incorporating Weather into Region-Wide Safety Planing Prediction Models. 85th TRB Annual Meeting, Washington DC., USA.

Sun, J., J. Sun, 2015. A dynamic Bayesian Network Model for Real-Time crash prediction using traffic speed conditions data. Transportation Research Part C 54 (2015) pp. 176-186.

Sun, X., H. Hu, E. Habib and D. Magri, 2011. Quantifying Crash Risk under Inclement Weather with Radar Rainfall Data and Matched-Pair Method, Journal of Transportation Safety & Security, 3:1, 1-14.

Treat, J. R., Tumbas, N. S., McDonald, S. T., Shinar, D., Hume, R. D., Mayer, R. E., Stanisfer, R. L., and Castillan, N. J. (1977). Tri-level Study of the Causes of Traffic Accidents. Report No. D.

Xu, C., P. Liu, W. Wang and Z. Li. Evaluation of the impacts of traffic states on crash risks on freeways. Journal of Accident Analysis and Prevention. Vol.47, 2012, pp.162-171.

Xu, C., P. Liu, W. Wang, and Z. Li. Predicting crash likelihood and severity on freeways with real-time loop detector data. Journal of Accident Analysis and Prevention, Vol. 57, 2013, pp. 30-39.

Yu, R., and M. Abdel-Aty. Utilizing support vector machine in real-time crash risk evaluation. Journal of Accident Analysis and prevention, Vol. 51, 2013, pp. 252-259.